Installation Declaration：

1. Read the manual carefully before installation and operate in accordance with the manual contents;

2. Installed by a qualified electrician. Confirm the power supply is turned off before installation and maintenance;

3. Suitable for indoor installation (not for humid environments);

4. The lamp shall not be directly connected to mains power 220V, but used with a driver as a set. For the dimming system used, conduct compatibility test with the driver in advance;

5. The lamp can be recessed installed on the ceiling surface, but do not cover heat insulators and combustibles above the lamp to avoid fire;

6. Do not install the lamp on an unstable or insecure ceiling to prevent the lamp from falling;

7. The light source of this lamp is non-replaceable. When replacement is required (e.g., at the end of the light source's service life), replace the entire lamp;

8. Keep this manual properly for future reference.

Two lead-end options:

1) bare wires (observe the +/- polarity labels when wiring);

2) with a quick DC connector.

Driver Selection：

| SLIR8FLD-N | WAC driver: 4LD-700MA10D-LF-C (0-10V) / to be selected by the customer based on the lamp's input parameters |
| --- | --- |
| SLIR8FLM-N | WAC driver: 4LD-350MA10D-LF-C (0-10V) / to be selected by the customer based on the lamp's input parameters |
| SLIR9FLD-N | WAC driver: 7EQA842001 / to be selected by the customer based on the lamp's input parameters |
| SLIR8FLD-M SLIR8FLM-M SLIR9FLD-M | WAC driver: 7EQA842001 / to be selected by the customer based on the lamp's input parameters |

Installation：

SLIR8*: Cutout diameter ￠45mm; SLIR9*: Cutout diameter ￠35mm, suitable for ceiling thickness 1-15mm.